QR Decomposition-Based

MVDR Beamformer

Versal FPGA Implementation using Vitis HLS

Platform: Versal VCK190  |  Tool: Vitis HLS 2024.1  |  Target: 400 MHz

# 1. Project Overview

This project implements a Minimum Variance Distortionless Response (MVDR) adaptive beamformer on the Xilinx Versal AI Core VCK190 FPGA platform using Vitis HLS. The covariance matrix inversion — the most numerically sensitive step in MVDR — is replaced by a QR decomposition-based triangular solve, yielding better numerical stability and a more regular, pipeline-friendly hardware structure.

Key Highlights:

- 8-element Uniform Linear Array (ULA) with 64 snapshots

- Fixed-point arithmetic: Q1.14 inputs, Q8.24 accumulators

- Householder QR on the sample covariance matrix R = XXᴴ/N

- Forward + back substitution to solve R⁻¹ aₛ without explicit inversion

- AXI4-MM interfaces mapped to Versal NoC (MC_NOC0)

- Target clock: 400 MHz on xcvc1902-vsva2197-2MP-e-S

# 2. Algorithm — MVDR via QR Decomposition

## 2.1 Classical MVDR

The MVDR beamformer finds the weight vector w that minimises output power while maintaining a unity gain in the look direction:

w = R⁻¹ aₛ / (aₛᴴ R⁻¹ aₛ)

where R is the M×M sample covariance matrix and aₛ is the M×1 steering vector towards the signal-of-interest (SOI).

## 2.2 QR-Based Reformulation

Rather than inverting R directly, we exploit the fact that R = X Xᴴ / N factors as R = (R_qr)ᴴ R_qr via QR decomposition of Xᴴ/√N. This replaces the ill-conditioned direct inverse with two triangular solves:

| 1 | Covariance matrix R = X Xᴴ / N (M×M Hermitian positive definite) |
| --- | --- |

| 2 | Householder QR Decompose R = (R_qr)ᴴ R_qr ⇒ R_qr is upper triangular |
| --- | --- |

| 3 | Forward substitution Solve (R_qr)ᴴ y = aₛ (lower-triangular system) |
| --- | --- |

| 4 | Back substitution Solve R_qr z = y (upper-triangular system) |
| --- | --- |

| 5 | Normalise w = z / (aₛᴴ z) to enforce distortionless constraint |
| --- | --- |

# 3. File Structure

| File | Type | Purpose |
| --- | --- | --- |
| mvdr_qr_beamformer.m | MATLAB | Golden reference: generates data, runs QR-MVDR, exports .bin test files |
| mvdr_qr.h | HLS C++ | Fixed-point typedefs (ap_fixed), array dimensions, function prototypes |
| mvdr_qr.cpp | HLS C++ | Full 5-stage kernel: compute_R → householder_qr → forward_sub → back_sub → normalise |
| tb_mvdr_qr.cpp | HLS C++ | C-simulation testbench: synthetic data, calls top-level, checks wᴴaₛ ≈ 1 |
| run_hls.tcl | Tcl | Vitis HLS project script: csim → synth → export .xo at 400 MHz |
| vitis_link.cfg | Config | v++ linker: kernel instance, AXI-to-NoC port mapping, profiling |
| host_mvdr_qr.cpp | C++ XRT | Host application: allocates BOs, fills data, launches kernel, verifies output |
| Makefile | Make | Full build flow: csim ¦ xo ¦ xclbin ¦ host ¦ run |

# 4. Fixed-Point Number Format

All arithmetic uses Xilinx ap_fixed<> types to match the DSP58/DSP58E4 cascades inside Versal PL:

| Signal | Type | Range | Notes |
| --- | --- | --- | --- |
| Array data X | ap_fixed<16,2> | [-2, 2) | Q1.14 — normalise to ±1 before loading |
| Steering vector aₛ | ap_fixed<16,2> | [-2, 2) | Unit-modulus complex exponentials |
| Accumulator (R) | ap_fixed<32,8> | [-128, 128) | Prevents overflow summing 64 products |
| Weight output w | ap_fixed<16,2> | [-2, 2) | Returned via AXI-MM gmem2 |

# 5. Key HLS Optimisations

## 5.1 Interface Pragmas

Three separate AXI4-MM master bundles are used so the memory controller can service all three transfers concurrently:

#pragma HLS INTERFACE m_axi port=X     offset=slave bundle=gmem0

#pragma HLS INTERFACE m_axi port=a_s   offset=slave bundle=gmem1

#pragma HLS INTERFACE m_axi port=w_out offset=slave bundle=gmem2

#pragma HLS INTERFACE s_axilite port=return bundle=control

## 5.2 Memory / Compute Pragmas

# 6. Build Flow

## 6.1 Prerequisites

- Vitis HLS 2024.1 (with Versal device support)

- Platform: xilinx_vck190_base_202410_1

- XRT 2024.1 installed on host Linux

- MATLAB R2023b or later (for .m reference only)

- gcc/g++ ≥17 for host compilation

## 6.2 Step-by-Step

- Run MATLAB script to generate golden .bin files and verify algorithm

- Run C-simulation:  make csim

- Synthesise HLS kernel:  make synth  (check Timing & Utilisation reports)

- Export kernel object:  make xo

- Link to xclbin (hardware build, ~45 min):  make xclbin TARGET=hw

- Compile host:  make host

- Execute on VCK190:  make run  or  ./host_mvdr_qr mvdr_qr_hw.xclbin

## 6.3 Quick C-sim Only

vitis_hls -f run_hls.tcl

# 7. MATLAB Reference (mvdr_qr_beamformer.m)

The MATLAB script serves three purposes:

- Algorithmic verification — compares direct MVDR (R\a) against the QR-based formulation; prints max weight error and SINR for both methods.

- Beampattern plotting — scans −90° to +90° and overlays both patterns, with markers at SOI (30°) and jammer angles (−20°, 50°).

- Fixed-point simulation — quantises data to Q1.14 and re-runs the algorithm to predict hardware SINR degradation before synthesis.

- Test data export — writes X_data.bin, a_steer.bin, and w_golden.bin for bit-accurate HLS testbench comparison.

Simulated scenario: 8-element ULA at 3 GHz, SNR = 10 dB, two jammers at JNR = 30 dB.

# 8. Verification Checklist

| Status | Check | Pass Criterion |
| --- | --- | --- |
| ☐ | MATLAB: max¦w_direct − w_qr¦ printed | < 1e-6 |
| ☐ | MATLAB: SINR difference between methods | < 0.1 dB |
| ☐ | C-sim: distortionless constraint ¦wᴴaₛ − 1¦ | < 0.05 |
| ☐ | HLS synth: worst-case timing slack | ≥ 0 ns at 400 MHz |
| ☐ | HLS synth: BRAM usage | < 50% available |
| ☐ | HLS synth: DSP usage | < 70% available |
| ☐ | Host run: distortionless error on hardware | < 0.10 |

# 9. Customisation

## Changing Array Size (M)

Edit  #define M 8  in mvdr_qr.h. Increase accumulator word-length if M×N_SNAP overflows Q8.24.

## Changing Snapshot Count (N_SNAP)

Must be ≥ M for a full-rank R. For N_SNAP not a power-of-two, replace the  >> 6  shift in compute_R with an ap_fixed divide.

## Targeting a Different Versal Device

Update  set_part  in run_hls.tcl and the platform name in Makefile and vitis_link.cfg.

## Adding AI Engine Offload

The NoC bridge is pre-allocated in vitis_link.cfg via the aie_pl_bridge comment. Un-comment and implement the AIE graph to offload the matrix-vector multiply (compute_R) to the AI Engine array for higher throughput at lower PL utilisation.

# 10. Known Limitations

- The complex division in the triangular solves uses hls::sqrt and float casts internally, which prevents full integer-only synthesis. If a purely fixed-point pipeline is required, replace with a CORDIC-based reciprocal.

- Co-simulation (cosim_design) requires an RTL simulator licence (Vivado Simulator or Questa). C-simulation alone is sufficient to verify functional correctness.

- The host application assumes device index 0. For multi-device servers pass the BDF string to xrt::device(bdf).

- Hardware build time on a typical workstation is approximately 45–60 minutes for the full VCK190 implementation.

Generated for Versal VCK190  •  Vitis HLS 2024.1  •  xcvc1902-vsva2197-2MP-e-S